T W D C F   v 1 . 3

===================

TWDCF is FREEWARE written by Thomas Waldmann, 2:247/720@fidnonet.org

Please send comments, bug reports etc. to this address. If this

address is not valid, please lookup in the FidoNet nodelist.

FUNCTION

========

This program can be used to set your DOS and realtime clock using

the signals from a simple DCF-77 receiver connected to a port bit

of your choice.

HARDWARE

========

You need some kind of DCF-77 receiver which provides a digital

output with the DCF-77 pulse signal. Connect this signal to a port

of your choice. I use the PaperEmpty signal of LPT2 (it is not used

in my system) which is pin 12 of the LPT SubD-connector at my

computer. The signal provided at this pin should be TTL-compatible.

The polarity doesn't matter because the program can invert the signal.

PROGRAM

=======

To get help about the usage of the program please enter  TWDCF -?

at the DOS prompt. You will get the following text:

--------------------------------------------------------------------------

DCF-77 decoder V1.2, 9.8.92, written by Thomas Waldmann, 2:247/720@fidonet.org

Usage: twdcf [-option]

Option {val} Description [Example]:

?           Help (no DCF-operations, but only -d possible)

d           set & save <D>efault settings

sS {1}      <S>et Clock (DOS&RTC) [-s0 = don't set, -s1 = set]

vV {0}      <V>erbose [-v0 = off, -v1 = on]

wW {1}      <W>indows Support [-w0 = off, -w1 = on]

pP {0279}   <P>ort Hex-Address [-p279 = 0279h = LPT2]

bB {5}      <B>it  0-7         [-b5   = Bit 5 = Paper Empty]

iI {1}      <I>nvert Portbit [-i0 = don't invert, -i1 = invert]

rR {1}      # of <R>etries [-r1 = 1 Retry on Failure]

xH,S,T,L    e<X>tended Settings [-x15,170,13000,10000]

{   15}      H min. ticks <H>igh for logical 1

{  150}      S min. ticks low for <S>ync

{13000}      T min. ticks for <T>imeout

{10000}      L tick <L>ength in microseconds

eE {45}     <E>xtra initial delay [-e45 = 45s] - useful it with -w1

--------------------------------------------------------------------------

Some comments on that:

-d      You can save the actual settings into twdcf.exe if you use this

option. Please change into the directory that contains twdcf.exe

before you use this switch. The program will patch itself with

the new settings.

-sS     You must use -s1 if you want to set your computer's clock

otherwise (-s0) nothing is changed.

-vV     If you use -v1 you'll get a verbose output of what's going on:

While waiting for a sync pulse (59th second) it shows only the

bit-value, the timer ticks the signal was high and the timer ticks

the signal was low:

B0 H11 L082 means bit=0, ticks_high=11, ticks_low=82

-x H   should be chosen half between the values of ticks_high and

ticks_low.

S*L should be the minimum time to recognize a sync signal,

which occurs every 59th second of a minute (missing H-pulse).

It should work with this value being about 1.7 seconds.

T*L is a timeout value if any signal is missing.

L   is the timing unit in microseconds - e.g. 10000 for a 10ms

timing unit.

After a sync pulse, it shows a line like this:

B<bit-value> H<high-ticks> L<low-ticks> HH:MM:SS DD.MM.YY WkD TZ

giving you the bit-value, ticks_high, ticks_low, the time (you

must subtract 1 minute, because the information transmitted is for

the FOLLOWING MINUTE !!!), the date, the day of week and the timezone.

The data is shown as it becomes available - that means it is complete

at the end of the minute after syncronizing.

Finally it shows the DCF-77 and your local DOS time to you.

-wW     If you run windows it is very likely that the program doesn't work

correctly if you use multitasking (enhanced mode /3). To work

around this use option -w1 which switches off multitasking while

decoding the DCF-77 signal. Use -w0 if you don't use TWDCF under

MS-Windows.

-eE     If you start this program event-driven (e.g. via a Mailer like

Frontdoor if you are a sysop) it'll start immediately after (!)

the begin of a minute - very bad because it must wait for the begin

of the next minute to synchronize. If you use -w1 this waiting time

will be lost and your system will be blocked.

This switch can be used to add an extra initial delay (non-

blocking!). If you had a blocking delay of a minimum of e.g. 47s

with version 1.1 of this program, you can now use -e45 and have a

non-blocking delay...

-pP and -bB:

P     B  Pin   Description

-------------------------------------------------------

201   4    2   gameport switch 1

201   5    7   gameport switch 2

201   6   10   gameport switch 3

201   7   14   gameport switch 4

379   7   11   LPT1 busy

379   5   12   LPT1 paper empty

279   7   11   LPT2 busy

279   5   12   LPT2 paper empty [DEFAULT]

3BD   7   11   LPT/HERC busy

3BD   5   12   LPT/HERC paper empty

3FE   6   22/9 COM1 ring indicator (25D/9D)

2FE   6   22/9 COM2 ring indicator (25D/9D)

Have a nice time! ;-)

Thomas

Changes:

========

1.0 to 1.1 : - a silly idiotic bug in -pP and -bB was removed,

now these options work like intended.

- option -d : program is able to patch itself with new

default values

- option -v : some cosmetic changes and explanation in

this text

- some other option : change them to have an 0/1 argument

meaning OFF/ON.

1.1 to 1.2:  - bug in date setting code fixed. date is now set, too.

- -eE option added

- some cosmetic changes

1.2 to 1.3:  - mysterious hangups in v 1.2 - hopefully corrected now

[End Of Text]